# User Manual

Table of Contents

Required equipment:	2

Installation	2

Simple associative conditioning:	2

Setting up the experiment	2

During the experiment	10

Data output	10

Extra functionalities	11

Settings and calibrations	11

Texture animation	15

Arduino connection	16

# Required equipment:

- Walking compensator with two mice optic detector at 90° of each other.

- Cylindrical or semi-spherical screen

- Video projector

- Computer

# Installation

Unzip the software where you want it to be installed and that’s it you can use it.

# Simple associative conditioning:

Have the walking compensator and the video projector plugged in before starting the software.

## Setting up the experiment

- Select Arena Shape

  - Open Arena is the recommended setting, a cylindrical arena 20cm in diameter

- Select which mice are the detectors

By moving the cursor, you should be able to see which mice is the computer connected one since it will change the value next to its corresponding mouse number. By elimination the detectors are the two other mice. To be sure which mice are the detectors just move your trackball and see what values are changing.

Click once on each mouse you want as detectors and then click Done.

NB: If you make a mistake in this step can click Re-select Mice to re-open the selection window.

- Chose two .png file for your stimuli

  - Click Browse > chose a file > click Select > click Load To List > repeat

For the picture 400x400 pixels works well with the default settings.

If you only want one stimulus you can load one normal texture and one transparent one, and then click Ignore This to keep the transparent object from triggering a choice.

- Chose the texture for the floor and the walls or the arena

  - Click Browse > chose a file > click Select > click Load Wall Texture

  - Click Browse > chose a file > click Select > click Load Floor Texture

Using a transparent texture works well to set a simple environment

- You can then click Inspect List to check if everything was loaded properly and how the environment looks

NB: pressing F11 twice switch the monitors so you can have the VR scene on your main monitor

Pressing F11 again will bring you back to the normal display. Click Inspect List one more time to stop the preview.

- Set your experiment’s protocol:

  - Enter how many different type of task there will be, for this exemple we’ll chose 1 learning phase and 1 test.

  - Fill in the parameters

US Duration: How long, in seconds, will the simulation freeze for the US delivery

CS Start: How long before the stimuli appear on screen and the bee can start moving in the VR. This is basically the inter trial interval.

CS Stop: How long before the trial end. Here 90s for CS Stop and 30s for CS Start means that a trial last for 60s.

Repetition: How many time should the line occur.

NB: You can click Copy To All to copy US Duration, CS Start, and  CS Stop on every line

Stim 1 and Stim 2: The name of the stimulations to use for this line, only useful if you’ve loaded more than two stimulations.

Test ?: Is this line a test? If No the simulation will freeze for US Duration seconds if the bee makes a choice. If Yes the simulation doesn’t freeze when the bee makes a choise.

PreTest ?: If Yes the software will compute which stimulus the bee spent the most time choosing and display its name in the lower right corner at the end of the trial.

Is 2D ? If selected this option will switch the VR to a 2D mode where the bee can only turn on the spot making the stimuli turn around her.

Once you’ve set your protocol you can click Hide to hide the parameter window.

To finalize the protocol you need to click Generate random Sequence of Stimuli, you can also click Show Sequences to see the sequences generated.

NB: This generates pseudo random sequences, following the rule that stimuli must not appear on the same side more than twice in a row.

- Save your protocol:

Enter the name of this protocol in the field next to Browse, click Browse to chose a folder and then Save to save your protocol.

If it works you should get this message

Protocols are saved as .txt files, meaning that you can edit them manually.

- Chose where to save the data from this experiment

Chose the name of the file, and click Browse to chose where to save it.

Upon starting the experiment, the software will create a file with a name following this format: Name_of_this_Exp_DATE_BEE-ID. Each time a full protocol is completed the BEE-ID will increase by one and a new file is created.

- Set the Latency

By default it’s set to 10s, meaning that the experiment will start 10s after you press start.

And you’re done, you can now click start and begin your experiment.

## During the experiment

Because all the mice control the pointer at the same time you might want to hide it by pressing F2 so that you don’t see it moving around on the screen as the bee walk. You can control the experiment via the keyboard to some extent. 
Space will start and pause the experiment.

CTRL+Space will stop the experiment.

SHIFT+Space will increment the Bee-Id by one.

# Data output

Data are saved as .csv files with semi-colon separators [;]

Excel or R should have no problem opening them, there might be some issue to open them on a Mac since it might expect colons and not semi-colons.

- Line: The number of the instruction line in your protocol. In our example above line 0 is the learning trials and line 1 is the test

- Trial: The number of the repetition within the line. In our example line 0 has 10 trials and line 1 has 1.

- Time(s): Time in seconds since the beginning of the experiment.

- PositionX/Z: coordinate of the bee on the horizontal plane.

- Rotation: Heading of the bee, 0 is forward.

- DistanceTotale: Total distance walked since the beginning of the trial.

- Speed: Mean speed between two data point, the point being very close to each other this should be close to the instantaneous speed of the bee, in m/s.

- Choice: Name of the chosen stimulus, if the bee is making a choice.

- Test: Is this a test, 0 for no and 1 for yes.

- Centered: What object is the bee centering on the screen.

- Looking_at: What object is close to the center of the screen.

- Edge: Is the bee centering the edge of and object and if yes which one.

- Edge_coord: Coordinate of the centered edge

# Extra functionalities

## Settings and calibrations

- Screen shape

You can set the shape of the screen you’re projecting on, by default it’s set to Cylinder, but you can also set it for a spherical screen or a flat one.

- Shape of the Stimuli

You can chose between Cylinder, Cube or Sprite, by default it’s set to Cube.

Sprite will display the stimuli on two flat surfaces that always face the camera.

- Size of the stimuli

You can change the size of the stimuli; the values are in meter.

- Recording frequency:

Time allows you to set the minimum duration in seconds between two data recording, leaving it to 0.0 means that the software records a data point every processor cycle (~every 0.0167 second).

- Test Distance

After choosing an Arena and setting your detector you can click Test Distance to see what mouse sensitivity you should use. Click on Test Distance and then have your walking compensator make a complete rotation along the X axis (forward/backward around the horizontal axis). If everything is set correctly the value in Distance should be the same as the one displayed in Distance expected in the upper left corner, if it’s not the software will suggest a new X sensitivity to correct the difference however before changing the sensitivity you should make sure that the Ball Radius is correct as it is the most likely source of error.

- Test Heading:

After choosing an Arena and setting your detector you can click Test Heading to see what mouse sensitivity you should use. Click on Test heading and then have your walking compensator make a complete rotation along the Y axis (left/right rotation around the vertical axis). If everything is set correctly the value in Heading should be the same as the one displayed in Angle expected in the upper left corner, if it’s not the software will suggest a new Y sensitivity to correct the difference however before changing the sensitivity you should make sure that the Ball Radius is correct as it is the most likely source of error.

- Mice calibration:

You can set the DPI of the mice you’re using in the walking compensator. You can also set the Radius of the ball you’re using, by default the software will expect a 2.45cm radius. The default Sensitivity should be fine on any setup but you might want to change it if your video projector is behind the bee (inverting the sign) or if you want to create discrepancies between the bee’s movements in the real world and the movements in the virtual one.

- Edge scale:

Edge is the scale of the edges of the cube relative to the cube size. Edges are used to detect if the bee is centering the edges of the cube or not.

- Calibrate Projector:

Displaying this target on your screen might help you position your video projector properly in relation to the screen. The target is deformed according to the selected screen shape.

## Texture animation

- Texture:

  - Target can be Wall or Floor for the wall and the floor texture respectively.

  - Tiling allows to set how many time the texture is repeated on each axis.

  - Offset allows to set the offset of the texture on each axis

  - Click Update to apply your changes

- Animation:

  - Offset increment allows to set the movement of the texture during each frame in pixels along each axis

  - Fps sets the number of time the texture is changed per seconds, this is capped by the number of processor cycle per seconds.

  - Update to apply your change.

  - Play to start the animation.

  - Stop to stop it.

## Arduino connection

- Serial Port: Name of the serial port to which the Arduino is connected, pressing Enter after typing the name will establish the connection

- Connect: This button is not working as intended in current version. See Serial Port entry above on how to establish a connection.

- Connection Status: Port_Name_Open if the port is connected and open, Port_Name_Close if the port is connected but close, None if no attempt as connection has been made, No port if no port is detected after connecting attempt.

- Ping: Will send “1” as a signal through the port

If an Arduino is connected to the setup the software will send “1” through the port each time an experiment starts and “0” each time it ends. Those features might be expended in the future.